SCOB032

Maake                       Khomotjo

Maake                       Mpho

Mabotja                    Rabbi Josephine

Mabunela                Bokamoso Justin

Machete                  Karabo Suzan

Madamalala          Nikhodani

Madia                       Mapiti Gift

Madlopha               Khayalethu Carlton

Madzivhandila      Mulalo

Maebela                  Kamogelo Clement

Maetane                  Thabang Botshelo Johannes

Magabjane              Kgotlello

Mahlalela                Teleki Amorn

Mahlamale             Muhluri

Mahlangu                Makabongwe Mthembeni

1.INTRODUCTION

The University of Limpopo is upgrading its network infrastructure to accommodate the growing need of students, staff, researchers and IoT devices. The project is about designing a secure, reliable, scalable and manageable campus network that supports high speed Wi-Fi, VoIP, Video streaming, research activities and smart devices.

The network will link the main campus in Polokwane to several buildings and a regional campus routers and gateway. A key focus will be on cybersecurity, to protect university data and systems from threats, such as ransomware, data breaches, and other cyberattacks. The project will also employ network simulation and some physical equipment for a practical and cost effective implementation.

2.Project Scope

The scope of this project is to design, implement and secure a modern multi-site smart campus network for the University of Limpopo. The project will include the main campus of the university in Polokwane, which will consist of Administration, Lecture Halls, Library, Student Residences and Research Laboratories and a satellite site connected through secure VPN. The network will be designed to support more than 500 simultaneous users, high-speed Wi-Fi, VoIP, video streaming and IoT devices such as smart classroom equipment, security cameras and environmental sensors.

The project will involve the creation of VLANs, IP addressing, routing, QoS, firewall zones, ACLs, VPN and Zero-Trust implementation. The development of the SDN integration plan to ensure the network’s optimal performance, scalability, automation, and security will also be accomplished. The designed network will be built and tested using Packet Tracer network simulators or limited physical equipment if possible. The testing will focus on the network’s connectivity, performance, failover capabilities, security, monitoring, and troubleshooting. The project will also produce the network documentation (diagrams, configurations, test results), recommendations, references, and scripts or configurations backups if applicable

3.Requirement Analysis

The University of Limpopo network will have to provide the necessary means to meet the communications and IT needs of an institution of higher learning. The network will be expected to give users dependable access to academic, administrative, and other services available on the network.

3.1 User Requirements

The main user needs that have been identified for the network include:

- Academic staff need access to university servers, learning management system, Internet, email, and network resources.

- Students need dependable wired and wireless access to learning resources, Internet, and university applications.

- Administrative staff need access to administrative systems, email, authentication services, and other shared resources.

- Network administrators and IT people will need a secure access method for managing network devices, servers, and network management systems.

- Guests and visitors should have Internet access via a separate guest wireless network and no access to internal university resources.

- Network services should have continuous connectivity among clients, servers, routing, and security equipment.

- Wireless users should have campus Wi-Fi access along with proper segregation of internal and guest traffic.

- The network should also be able to host other services like VoIP, video conferencing, and IoT.

Thus, VLAN Segmentation technique was used while designing the network.

3.2 Hardware Requirements

This suggested network will need a combination of Layer 3 switches, Layer 2 access switches, servers, wireless, and security appliances.

The hardware requirements are as follows:

| Hardware | Use |
| --- | --- |
| Core Layer 3 switches | Fast campus backbone and inter-VLAN routing |
| Distribution switches | Summary of access networks and routing |
| Access switches | Connection to PCs, servers and other endpoint devices |
| Server switches | Exclusive connectivity for the server infrastructure |
| Servers | DNS, DHCP, LMS, AAA, email, SDN and others |
| Wireless controllers | Central management of the wireless network |
| Access Points | Provide Wi-Fi connectivity on campus |
| Firewall | Security perimeter separating internal network from external network |
| User devices | Access by students and staff |
| VoIP devices | Future/present implementation of voice communication |
| IoT devices | To support smart-campus future services |
| Cables | Copper Ethernet cables connecting network devices |
| WAN/Internet devices | Connectivity to external networks and satellite campuses |

Redundancy in core/distribution connectivity is critical in order to ensure continued operation of the network if one of the paths becomes inaccessible.

3.3 Software Requirements

The network will need software that is capable of offering routing services, addressing, authentication services, name resolution, management, and applications services.

The major software needs of the network are:

- Cisco IOS used in configuring routers and layer 3 switches

- OSPF protocol for dynamic routing services among layer 3 network devices

- DHCP service for automatic allocation of IP addresses to end devices

- DNS service to resolve university hostnames to IP addresses

- AAA/authentication services for authentication and authorization of users

- Web/LMS services for online learning

- Email services for university communications

- FTP for controlled file transfer and network management/administration testing.

- Wireless controller software for wireless network and access point management.

- Firewall/software for controlling the flow of data in trusted and untrusted networks.

- Network management/monitoring software for monitoring of network devices and services.

- Cisco Packet Tracer for simulation, configuration and testing of the proposed network.

We have successfully set up and tested the following DNS names during implementation:

- lms.unilim.local

- aaa.unilim.local

- sdn.unilim.local

- mail.unilim.co.za

DNS resolution was tested from client devices.

3.4 Bandwidth Requirements

The network will have to accommodate adequate bandwidth for routine operations of the university as well as for other applications that might generate higher bandwidth needs.

These include:

- Web browsing

- Online education/LMS access

- Email

- File transfer

- DNS and DHCP traffic

- Authentication

- Video conference

- Voice over IP

- Wireless Internet connection

- Server-to-server communication

- Network management

- Internet of Things

Thus, the backbone as well as the distribution layer will need to have higher bandwidth capacity than individual end-user connections.

Design considerations include:

- End devices → Access Switch → Distribution layer → Backbone →    Firewall/WAN/Internet

- High-speed links between the core and distribution layers are required to prevent the backbone from becoming a bottleneck.

- For wireless networks, bandwidth must also be shared among multiple simultaneous users. The design therefore separates internal wireless traffic from guest wireless traffic using VLANs.

3.5 Security Requirements

Security becomes a requirement due to the presence of students' data, staff data, authentication data and institutional services over the university network.

There must be provision of:

Network Segmentation

VLANs have been made for the various purposes of the network. The VLAN structure adopted in this project consists of:

| VLAN | Name | Purpose |
| --- | --- | --- |
| 10 | MANAGEMENT | Network-device management |
| 20 | ADMINISTRATION | Administration users |
| 30 | ACADEMIC-STAFF | Academic/staff users |
| 40 | STUDENTS/LECTURE | Student and lecture-related network |
| 50 | LIBRARY | Library users and services |
| 60 | RESIDENCES | Student residences |
| 70 | RESEARCH | Research users and laboratories |
| 80 | SERVERS | University server network |
| 90 | VOICE | VoIP |
| 100 | CAMPUS-WIFI | Internal wireless users |
| 110 | GUEST-WIFI | Guest wireless users |
| 120 | CCTV | Security cameras |
| 130 | IOT | IoT devices |
| 140 | ENV-SENSORS | Environmental sensors |

The network will use firewalls to regulate traffic between internal and external networks. The AAA server will offer the services of authentication, authorization, and accounting for users and admins. Guest Wi-Fi will be separated from internal university networks, whereas critical servers will be used on a separate server VLAN. Access to the devices will only be provided to authorized IT personnel. Malware and ransomware will be mitigated by segmentation, access controls, authentication, backups, and monitoring.

3.6 Scaling Requirements

The network at the university has to support expansion in the future.

The design has to be able to:

Support more students and staff.

Support more PCs and laptops.

Support more wireless access points.

Support more servers.

Support more VLANs.

Expand VoIP capabilities.

Expand video-conferencing capabilities.

Support more IoT devices.

Support construction of additional buildings on the campus.

Expand to other campuses.

Support larger Internet bandwidth.

Support more network management and security systems.

With VLANs and dynamic routing via OSPF, it becomes possible to expand the network without configuring routes for each individual device.

4. Network Design

4.1 Network Architecture Proposal

The proposed network for the University of Limpopo will have a hierarchical network architecture which consists of core, distribution, access/server, security, and service layers. It is proposed that the network can accommodate more than 500 users and ensure reliable connectivity, network segregation, redundancy, security, and centralized services.

The two multilayer switches that form the core of the network are CORE-SW1 and CORE-SW2. The switches form the backbone of the network and use dynamic routing using Open Shortest Path First (OSPF).

DIST-SW1 and DIST-SW2 are the components of the distribution layer of the network. These switches are connected with the core network and link it with different segments of the network and give routing and connectivity towards the access and server networks.

ASW-SERVER1 is the component used for the server network. The server network includes various services such as DNS, DHCP, LMS, authentication, email, FTP, and SDN related services.

MAIN-FW1 firewall is placed at the edge of the network for providing security between the internal university network and other external networks. Wireless connectivity is provided through wireless infrastructure.

The overall architecture can therefore be represented as:

Users/Wireless → Access Layer → Distribution Layer → Core Layer → Firewall/External Network

with the server environment connected through the distribution/core infrastructure.

4.2 Network topology

A packet tracer topology follows a hierarchical network design, with a number of links between the core and distribution layers for redundancy. The following switches represent the key layers within the network:

The core switches are:

CORE-SW1

CORE-SW2

The distribution layer includes:

DIST-SW1

DIST-SW2

The server/access layer includes:

ASW-SERVER1

The firewall is:

MAIN-FW1

The topology implements several connections between the core switches and the distribution switches, allowing the traffic to be routed using alternative routes instead of one physical connection.

OSPF is implemented between the Layer 3 switches. The router IDs applied in the configuration of OSPF are as follows:

| Device | OSPF Router ID |
| --- | --- |
| CORE-SW1 | 1.1.1.1 |
| CORE-SW2 | 2.2.2.2 |
| DIST-SW1 | 3.3.3.3 |
| DIST-SW2 | 4.4.4.4 |

The core switches provide redundant paths to the distribution layer. During the testing, OSPF neighbors reached FULL status, which means that OSPF adjacencies were created successfully. It increases the network availability as traffic can be re-routed through another route.

4.3 IP Addressing Scheme

The network uses a structured approach to IP addressing, using a number of subnets to allow the network to be segmented and provide routing at Layer 3. The network supports VLANs and allows Layer 3 devices to forward packets between network segments.

The address scheme also provides addresses for network gateways, servers, infrastructure devices and hosts. Where appropriate DHCP is used to ease the configuration of client addresses.

4.4 VLAN Design

VLANs allow networks to be logically segmented into distinct groups. VLANs reduce traffic on shared media and provide increased levels of security and organisation to the network.

The network makes use of VLANs to segment the network into a number of areas of operation, including:

VLAN 10 – Management network

VLAN 20 – Administration network

VLAN 90 – Voice network

The trunk links between switches carry the VLANs, and the access ports are associated with a particular VLAN, depending on which devices are connected.

4.5 Routing Design

Routing in the network takes place at Layer 3, using the core and distribution switches. The dynamic routing protocol OSPF has been used between the Layer 3 switches.

OSPF allows the routers and Layer 3 switches to send updates to each other, determining which routes are available. The use of multiple links between the core and distribution switches provides route redundancy, as an alternative link can be used if a connection is lost.

4.6 Wireless Network Design

The wireless network provides access to the network for mobile devices, such as laptops and other wireless clients. Wireless access points provide wireless connectivity and are connected into the backbone switching infrastructure, providing users with access to the relevant wireless network/VLAN.

Where appropriate traffic from wireless networks is segregated from other traffic using VLANs and the appropriate addressing. This allows wireless users to access network services that they are permitted to use, with the capability to logically separate users from other departments, if required.

4.7 Server Design

The servers are connected into the network via the server/access layer, providing several services to the network. These services can include DNS, DHCP, web/application services and other infrastructure services as required.

Servers are placed on dedicated network segments for the purpose of management and security. Access to resources on these servers are controlled by VLANs, routing and firewall rules where appropriate.

4.8 VPN Design

The VPN design allows users to securely access the network infrastructure from a remote location. The connections can be terminated at the firewall or at a designated VPN gateway, allowing remote users to access the required resources

The connections should make use of authentication and encryption to ensure that traffic between remote users and the network is kept secure.

4.9. IPsec VPN Design

The IPsec VPN design was formulated to establish a secure connection between the various network segments while also ensuring reliable communications between network sites. The design makes use of the Internet Protocol Security (IPsec) Virtual Private Network (VPN) technology to secure traffic traversing over an untrusted network. The network under consideration already has a hierarchical design which includes the core, distribution, and access switches in addition to the VLAN segmentation and dynamic routing protocol such as the Open Shortest Path First (OSPF). As such, the IPsec design should be considered as an addition to the current network infrastructure rather than a replacement. The use of separate VLANs and routed networks allows for logical segregation of the traffic whereas the IPsec can be deployed to establish a secure tunnel between the necessary network gateways. The design also considers the load balancing and redundancy principles that were applied in the previous sections. In this case, the Hot Standby Router Protocol (HSRP) and Open Shortest Path First (OSPF) can be considered for providing an appropriate gateway and alternative routes respectively. This implies that the proposed design can be integrated into the overall network model to enhance both security and network availability.

4.10.  802.1X Design

The 802.1X design was applied on the ASW-ADMIN1 switch to provide port-level security for the Administration VLAN. The switch utilized the 802.1X protocol to ensure that only authorized devices can access the network through the ports of the switch. The 802.1X implementation was configured with the RADIUS server as an external authentication system as opposed to using local authentication, which was set to 10.10.9.102. The Administration switch had two uplink trunk connections to DIST-SW1 and DIST-SW2 switches. However, the user access ports were set to the VLAN 20. In the initial 802.1X configuration and testing, the FastEthernet0/1 port was chosen for use with the Admin PC1 as the authentication-controlled port. Initially, the FastEthernet0/1 interface was set as an access port with VLAN 20, and the mode was set to authenticator in the 802.1X configuration.

5.Cybersecurity Design

5.1 Firewalls

The firewall used by the network is that of Cisco ASA. It is used to act as the security barrier between the University's network and the external networks.

The firewall acts as the barrier between the University's network and the external or Internet network. It prevents open access to the university network from external networks. Traffic control is done through routing and access control policies. The firewall is one of the components of the security design along with VLAN segmentation, ACLs, and Authentication.

5.2 Access Control

Access control is implemented by VLANs, switch port configuration and ACLs. Different categories of users and devices are placed in different VLANs which restrict their access to specific areas of the network.

The main VLAN structure includes:

| VLAN | Purpose |
| --- | --- |
| VLAN 10 | MANAGEMENT - Network-device management |
| VLAN 20 | ADMINISTRATION - Administration users |
| VLAN 30 | ACADEMIC-STAFF - Academic/staff users |
| VLAN 40 | STUDENTS/LECTURE - Student and lecture-related network |
| VLAN 50 | LIBRARY - Library users and services |
| VLAN 60 | RESIDENCES - Student residences |
| VLAN 70 | RESEARCH - Research users and laboratories |
| VLAN 80 | SERVERS - University server network |
| VLAN 90 | VOICE - VoIP |
| VLAN 100 | CAMPUS-WIFI - Internal wireless users |
| VLAN 110 | GUEST-WIFI - Guest wireless users |
| VLAN 120 | CCTV - Security cameras |
| VLAN 130 | IOT - IoT devices |
| VLAN 140 | ENV-SENSORS - Environmental sensors |

Access control lists were also set up on sensitive networks. Specifically, the Guest and IoT networks are restricted to prevent these devices from communicating freely with protected internal resources.

5.3 Network Segmentation

One of the main security controls used in the project is network segmentation.

The university network is segmented into various VLANs based on user type, service, or device. For example, separation of administration users, staff wireless users, students, guests, servers, voice and IoT devices.

This design minimises the impact of a security incident. For example, if a guest device or an IoT device is compromised, it is not automatically given unfettered access to administration systems, servers or network-management devices.

Security policies and Layer 3 switches control communication between VLANs.

The project also includes segmentation at the regional/satellite campus, with separate VLANs for users, voice and other services.

5.4 IDS/IPS

A dedicated IDS/IPS appliance was not implemented in the Packet Tracer simulation. Detection and protection were instead supported by firewall ACLs, VLAN segmentation, Zero-Trust ACLs, authentication, and centralized Syslog monitoring. A dedicated IDS/IPS can be added in a future physical deployment for deeper traffic inspection and automated threat detection.

5.5 Authentication

Authentication is provided to stop unauthorised users from accessing protected network resources.

The network design includes a AAA server to provide Authentication, Authorisation and Accounting (AAA). This provides a central control for administrative access to network devices.

Wireless networks are also secured using WPA2-PSK. Configured wireless networks are:
STAFF WIFI

- CAMPUS WIFI

- GUEST WIFI

- STAFF WIFI

- IOT-WIFI

The WLC manages the wireless networks and security settings.

Also, administrative interfaces are isolated from the regular user networks to reduce the chances of unauthorised management access.

5.6 Protection Against Ransomware and Data Breaches

The network incorporates multiple controls that mitigate the possibility of any impact and exposure to ransomware and data breaches.

Most importantly, the network segmentation is applied. Administration, servers, user, guest, voice, and IoT devices are placed in separate VLANs.

ACLs prevent the unnecessary communications between these networks. Most importantly, the communication of guest and IoT devices is limited to protected internal devices.

The firewall is yet another security boundary between the internal network of the university and the external network.

The access control and authentication controls are used for limiting the access to network management and protected resources.

Monitoring/logging is also provided in order to help the administrators identify suspicious activity on the network.

It should be noted that the above controls do not prevent the occurrence of ransomware or data breach. They simply limit the access and spread of any infection.

5.7 IoT security

IoT devices are isolated in VLAN 130 and protected by the IOT-ZT access control list. The policy permits communication with the authorised IoT server at 10.10.9.105 and required local services, while denying unnecessary access to protected internal campus networks. Testing confirmed permitted traffic to the authorised server and blocked lateral access to protected internal destinations.

5.8. IPsec VPN Security

The security of the IPsec VPN relies on keeping data safe while it travels between network locations. Ipsiс provides confidentiality, integrity, authentication, and protection against modification of network traffic. Encryption prevents unauthorised users reading the information passing through the VPN tunnel, while authentication allows the VPN endpoints to verify that they are communicating with the correct peer. Integrity checking helps spot if packets have been tampered with during transmission. In the network discussed, security is already an important part of the overall design, as demonstrated by the implementation of RADIUS and 802.1X authentication on the access switch. RADIUS was configured using the server address 10.10.9.102, with the configured authentication key, and 802.1X was enabled globally and on the selected access port. IPsec would provide an additional security layer at the network level, protecting traffic between the VPN endpoints rather than just controlling which individual devices are allowed onto the LAN.

5.9.  802.1X Security

802.1X was configured to provide an additional level of security for the Administration network by ensuring that only authorized computers could gain normal access to the protected switch port. The switch under consideration acted as the 802.1X authenticator, while the connected PC was the supplicant, and the RADIUS server (IP address 10.10.9.102) was used to process the authentication request. In other words, the switch was configured to delegate the authentication task to the RADIUS server, thus adding an extra level of security. The command aaa new-model and aaa authentication dot1x default group radius were necessary to establish this configuration, as they defined the authentication server to be used for 802.1X. The shared secret <REDACTED> was set on the switch to ensure the integrity of the communication between the authenticator and authentication server. Finally, the port was placed in the authentication port-control auto mode, which required it to participate in the 802.1X authentication process rather than allowing all traffic by default.

6. Implementation

6.1 Simulation Tool Used

Implementation of the network design has been carried out through use of Cisco Packet Tracer.

Cisco Packet Tracer was chosen because it allows the project to virtually simulate the existence of routers, multilayer switches, access switches, firewalls, WLAN controllers, access points, servers, IP phones, and IoT without necessitating the use of numerous networking hardware equipment.

The simulation enabled implementation of the university's VLANs, routing, DHCP, wireless services, VoIP, security, and inter-site connectivity.

6.2 Device Configuration

The network is made up of routers, multilayer switches, access switches, a firewall, a Wireless LAN Controller, wireless access points, servers, IP phones and IoT devices. Devices were appropriately configured with IP addresses, VLAN configurations, routing and security settings.

Main Network Infrastructure:

- Edge routers for connections to external and regional campus networks

- Core/ distribution multilayer switches

- Access switches for end user devices

- Cisco ASA firewall

- Wireless LAN Controller

- Wireless access points

- CME router for VoIP services

- DHCP/DNS and other servers

- IP phones

- IoT devices

6.3 VLAN configuration

VLANs were configured to separate different types of network traffic. Access ports were assigned to the appropriate VLANs while trunk links were configured between switches to carry multiple VLANs.

The main VLAN configuration includes:

| VLAN | Name |
| --- | --- |
| VLAN 10 | MANAGEMENT - Network-device management |
| VLAN 20 | ADMINISTRATION - Administration users |
| VLAN 30 | ACADEMIC-STAFF - Academic/staff users |
| VLAN 40 | STUDENTS/LECTURE - Student and lecture-related network |
| VLAN 50 | LIBRARY - Library users and services |
| VLAN 60 | RESIDENCES - Student residences |
| VLAN 70 | RESEARCH - Research users and laboratories |
| VLAN 80 | SERVERS - University server network |
| VLAN 90 | VOICE - VoIP |
| VLAN 100 | CAMPUS-WIFI - Internal wireless users |
| VLAN 110 | GUEST-WIFI - Guest wireless users |
| VLAN 120 | CCTV - Security cameras |
| VLAN 130 | IOT - IoT devices |
| VLAN 140 | ENV-SENSORS - Environmental sensors |

6.4 Route Configuration

Inter VLAN routing was done using multilayer switches.

Configurations such as SVIs were set up for the required VLANs and IP routing was enabled to ensure authorised traffic flow from one VLAN to another.

There is also dynamic routing using OSPF, which is enabled in the network infrastructure to enable routers and layer 3 switches to exchange network data.

The regional campus has its routing infrastructure and it connects to the main campus through the WAN.There is also a default route towards the external network when needed.

6.5 DHCP and DNS

The DHCP protocol has been developed to automatically supply IP configurations to client devices.

DHCP pools have been defined for all the required networks and give clients the following:

- IP address

- Subnet mask

- Default gateway

- DNS server details

There is a DHCP pool defined separately for the VoIP network.

Option 150 has been defined in the voice DHCP pool on the CME router to give IP phones the address of the TFTP/CME server so that the phones can retrieve their configurations.The DNS services are provided by the defined university DNS server, enabling client devices to resolve network services by names rather than IP addresses.

6.6 WI-Fi Configuration

The wireless network on the campus was deployed using a Cisco Wireless LAN Controller (WLC) and wireless access points. There are four separate Wi-Fi networks:

STAFF-WIFI for staff members

CAMPUS-WIFI for students and other users in the campus

GUEST-WIFI for guests

IOT-WIFI for IoT devices

Every wireless network has its own VLAN and IP network. This makes it easy to distinguish between different user and device types.

The WLC is configured with the management IP address 10.10.9.70. The wireless network is configured to use WPA2-PSK which prevents unauthorized users from accessing the network. The WLC and access points were configured to provide wireless coverage across the campus.

6.7 VPN Configuration

A site-to-site IPsec VPN connection was established between the primary campus and the regional or satellite campus.

The role of the VPN is to provide the connection for the encrypted traffic between the two campuses through the WAN.

The reason for configuring the VPN connection is to ensure the traffic that is flowing between the two campuses is not transmitted freely without any protection in the external network.

The reason for configuring the VPN connection is to ensure the traffic that is flowing between the two campuses is not transmitted freely without any protection in the external network.

The configuration of the VPN connection includes:

- IPsec

- AES encryption

- Pre-sharing authentication

- VPN access control/interesting traffic

6.8 QoS Configuration

Quality of Service (QoS) was configured for WAN voice traffic on MAIN-EDGE-R1. The WAN-QOS policy was applied outbound on GigabitEthernet0/0. The VOICE-RTP class uses strict priority with 20% bandwidth and DSCP EF, and ACL 120 identifies UDP RTP traffic in the range 16384-32767. Voice endpoints use VLAN 90, while data devices remain in their assigned data VLANs. Verification confirmed that the QoS policy and service-policy attachment were present. During the active inter-site call, Packet Tracer did not increment the VOICE-RTP packet counter, so live RTP classification was not demonstrated by the simulator.

6.9 SDN Controller Configuration

The UL Smart Campus SDN Controller was configured using Python in Packet Tracer for centralised monitoring and policy management. The controller used IP address 10.10.9.107 and maintained an inventory of key devices, including the DHCP/DNS server, WLC-MAIN, access switch, distribution switches and core switches. The program monitored connectivity and represented security policies for Management, Servers, Guest Wi-Fi, IoT, CCTV and Voice. An HTTP API was also used as the controller integration point because Packet Tracer does not provide full enterprise SDN/OpenFlow support.

6.10.  Redundancy and Failover Configuration

Redundancy and failover were configured and tested using HSRP (Hot Standby Router Protocol), which provides gateway redundancy between the distribution switches. For VLAN 40, the HSRP configuration had the virtual IP address of 10.10.4.1. This allows for hosts to be connected using a single default gateway, even if one of the distribution switches fails. In the HSRP status, DIST-SW1 is the Active router with a priority of 110, while Standby router is 10.10.4.3 with a priority of 100, allowing for preemption when the active switch fails. The configuration is illustrated using show standby command, which also displays the active/standby relationship in VLAN 40 as well as the virtual MAC address 0000.0C07.AC28.

On a network level, we can be sure that the redundancy is properly configured by checking the underlying network redundancy using OSPF. The CORE-SW1 switch has FULL OSPF adjacencies with both 3.3.3.3 and 4.4.4.4, and the routing table shows multiple equal-cost OSPF routes through both 10.10.255.10 and 10.10.255.14 for several VLAN networks. In addition, we have tested the connectivity to the HSRP virtual gateway 10.10.4.1 from the CORE-SW1 switch, which responded with five replies out of five, i.e., 100% packet success rate. This verifies that the gateway and routing redundancy are properly configured, allowing for traffic to take a different path if one of the switches or links fail.

6.11 Security Configuration

Security was done through several layers rather than depending on one security layer alone.

Security measures include:

6.11.1 Firewalls:

Firewall implementation was performed using a Cisco ASA 5505 device (MAIN-FW1) to provide security and access control between the main campus network and external networks. The process involved configuring the inside and outside interfaces, establishing routing, applying an access control list (ACL) and saving the final configuration.

6.11.1.1 Configuration of the Inside Interface

Inside interface of the firewall was configured using VLAN 1 and assigned the inside security zone. Security level of 100 was defined to mark the interface as the trusted internal network.

interface Vlan1

name if inside

security-level 100

ip address 10.10.255.1 255.255.255.248

The inside interface was assigned an IP address of 10.10.255.1/29 to provide communication between the firewall and the internal network.

6.11.1.2 Configuration of the Outside Interface

VLAN 2 was configured to serve as the outside interface and was assigned the outside security zone. Security level 0 was set to represent the untrusted external network.

interface Vlan2

nameif outside

security-level 0

ip address 172.16.1.2 255.255.255.252

The physical interface connected to the external router was assigned to VLAN 2:

interface Ethernet0/0

switchport access vlan 2

These configurations allow the firewall to treat all incoming traffic as external traffic.

6.11.1.3 Routing Configuration

A static route was created to give the connection to the main campus network:

route inside 10.10.0.0 255.255.0.0 10.10.255.2

This route sends all the traffic destined to the 10.10.0.0/16 network via the internal next hop address 10.10.255.2.

A default route was also created for forwarding the traffic to the external router:

route outside 0.0.0.0 0.0.0.0 172.16.1.1

The default route sends all the traffic which does not have an explicit entry in the routing table to the 172.16.1.1 router.

6.11.1.4 Access Control List Configuration

An extended ACL named OUTSIDE-IN was created for controlling the traffic coming into the main campus network from the external network.

access-list OUTSIDE-IN extended permit ip 10.20.0.0 255.255.0.0 10.10.0.0 255.255.0.0

The rule permits IP traffic from the 10.20.0.0/16 network accessing the main campus network 10.10.0.0/16 network.

The ACL is applied to the inbound traffic of the outside interface:

access-group OUTSIDE-IN in interface outside

Initially, there was a rule which permitted ICMP traffic from any external network but it was deleted since it provides unrestricted access to the main campus network:

no access-list OUTSIDE-IN extended permit icmp any 10.10.0.0 255.255.0.0

6.11.1.5 Saving the Firewall Configuration

Following the successful configuration process, the running configuration was saved to safeguard the firewall configuration from being lost upon restarting:

write memory

Thus, the firewall implementation was complete through the configuration of security zones, routing, access control, and saving of the resultant configuration.

6.11.2 VLAN Segmentation:

Users, servers, guests, voice and IoT are segmented through VLANs.

6.11.3 ACLs:

Access Control Lists restrict the traffic from sensitive networks. Specifically, guest and IoT networks are limited from entering protected internal networks.

6.11.3.1 ACL Configuration and Implementation

The ACLs have been configured on Layer 3 VLAN interfaces of the distribution switches to regulate the traffic between Guest Wi-Fi, CCTV, IoT and internal networks. Extended ACLs were configured and applied in inbound direction on VLAN 110, VLAN 120 and VLAN 130. Each ACL is configured to allow DNS communication to DNS server 10.10.9.100 while restricting any access from the respective network to the internal 10.10.0.0/16 network. In addition, IoT ACL allows communication to authorised host 10.10.9.105. The ACLs are applied to the VLAN interfaces using ip access-group command.

6.11.3.2 Guest Wi-Fi ACL – VLAN 110

For VLAN 110, the extended ACL named GUEST-ZT was configured and applied inbound to the VLAN interface. It permits DNS communication from Guest Wi-Fi network to 10.10.9.100, denies access from Guest network to internal 10.10.0.0/16 network and allows other traffic which do not match the defined restrictions.

6.11.3.3 CCTV ACL – VLAN 120

An extended ACL, namely CCTV-ZT, was set up and applied to VLAN interface 120 (inbound). The ACL allows requests to the DNS server from the CCTV network at 10.10.9.100 and blocks traffic to the internal 10.10.0.0/16 network. The ACL is enabled with the use of the command ip access-group CCTV-ZT in. VLAN 120 was added to the list of allowed VLANs on the trunk interface to allow CCTV traffic between switches during the ACL application.

6.11.3.4 IoT ACL – VLAN 130

An extended ACL, namely IOT-ZT, was set up and applied to VLAN interface 130 (inbound). The ACL allows requests to the DNS server from the IoT network at 10.10.9.100, allows communication with the authorised host at 10.10.9.105, and blocks access to the internal 10.10.0.0/16 network. The ACL is applied with the help of the command ip access-group IOT-ZT in, and provides network segmentation for the IoT devices.

6.11.3.5 Verification

After ACLs and VLAN setup, ACLs and VLAN connectivity were tested by the means of commands such as show access-lists, show running-config interface Vlan110, show running-config interface Vlan120, show running-config interface Vlan130, and show interfaces trunk. Network connectivity was tested via ping, and ARP verification using show arp were also performed to confirm that the ACLs and VLANs were operating as intended.

6.11.3.6 ACL Summary

| VLAN | Purpose | ACL | Reason |
| --- | --- | --- | --- |
| 110 | Guest Wi-Fi | Guest-ZT | Guests should not access internal campus networks |
| 120 | CCTV | CCTV-ZT | Cameras should have restricted internal access |
| 130 | IoT | IoT-ZT | IoT devices should only reach required services |

6.11.4 Wireless Security:

The WLC WLAN uses WPA2-PSK Authentication. Wireless security was achieved through the use of network segmentation and centralised wireless management. The campus design separated normal campus wireless users and Guest Wi-Fi by assigning them to different VLANs, with VLAN 100 being used for Campus Wi-Fi and VLAN 110 being used for Guest Wi-Fi. The SDN security policy specifically required Guest Wi-Fi to have access to Internet/DNS/DHCP only, while internal campus resources were denied. The WLC-MAIN was also included in the centralised device inventory and successfully monitored by the SDN controller. This approach provides controlled wireless access and prevents guest users from directly accessing protected internal campus networks. The available testing evidence affirms VLAN-based wireless security and guest isolation; it however, does not provide sufficient evidence to claim a specific wireless encryption or authentication method such as WPA2-Enterprise or WPA3.

6.11.5 AAA:

AAA Server is part of the solution in order to facilitate authentication, authorization and accounting.

6.11.6 Management Separation:

Separation of management traffic from other user traffic is facilitated through management VLANs.

6.11.7 IoT Isolation:

IoT devices are hosted in VLAN 130 and are limited from accessing internal University networks unnecessarily.

6.11.8 Monitoring and Logging:

The network has management/monitoring capabilities like Syslog and SNMP configuration which helps in monitoring the network activities.

6.11.9 VoIP Security:

Voice devices are segmented into VLAN 90 rather than being in the same VLAN as other users.

6.12 IPsec VPN Implementation

A site-to-site IPsec VPN was implemented between the main campus network (10.10.0.0/16) and the satellite campus network (10.20.0.0/16), with remote peer 172.16.0.6. The VPN was integrated with the existing routed network and tested on MAIN-FW1. The IKE security association reached QM_IDLE, confirming that the site-to-site tunnel was established. IPsec counters showed encrypted and decrypted traffic with no send or receive errors, and stable inter-site reachability was confirmed by successful pings to the satellite site. The VPN therefore provides protected communication between the two campuses while operating with the existing OSPF and HSRP-based network infrastructure.

6.13 802.1X Implementation

The implementation was done on ASW-ADMIN1 using Cisco IOS commands supported by the Packet Tracer switch. Initially, the command aaa new-model was used to enable AAA. Then, the RADIUS server was configured using 10.10.9.102 with authentication port 1645 and shared key <REDACTED>. The switch was set to use RADIUS for 802.1X with aaa authentication dot1x default group radius. The global 802.1X processing was allowed using dot1x system-auth-control. On FastEthernet0/1, where the Admin PC1 was connected, the port was set to dot1x pae authenticator and authentication port-control auto. The existing VLAN configuration was kept, and Fa0/1 was set to the access port with VLAN 20 (ADMINISTRATION). The running configuration was checked to ensure that the AAA, RADIUS, global 802.1X processing, and the required port 802.1X settings were correctly set. The switch did not allow using the command show dot1x verification, so the verification process was done by looking at the running configuration and interface configuration.

6.14 NAT/PAT Implementation

Dynamic PAT was configured on MAIN-FW1 to translate ordinary campus traffic leaving the internal network. A CAMPUS-NET network object was created for 10.10.0.0/16 and configured with nat (inside,outside) dynamic interface. Verification with show nat confirmed the rule was active, and translation hits increased during testing, demonstrating that the firewall was processing PAT traffic. The site-to-site VPN was retested after PAT was added and remained operational in the Packet Tracer simulation.

6.15 SDN Controller Implementation

The UL Smart Campus SDN Controller was implemented using Python in Packet Tracer to provide centralised monitoring and policy management for the campus network. The controller was assigned the IP address 10.10.9.107 and maintained a central inventory of important network devices, including the DHCP/DNS server, wireless controller, access switch, distribution switches, and core switches. The Python program also monitored connectivity and defined SDN security policies for important VLANs, including Management, Servers, Guest Wi-Fi, IoT, CCTV, and Voice. An HTTP API was also implemented to represent the controller integration point and expose the central policy information.

7. Testing and Results

7.1 Connectivity Testing

Connectivity was tested network-wide to ensure proper communication between the core, distribution, access switches, VLANs, and end devices. Overall, the network was properly connected across all the VLANs and routing infrastructure. On CORE-SW1, OSPF neighbours were discovered with router IDs 3.3.3.3 and 4.4.4.4, both of which were in FULL neighbour state. This shows that the core switch was properly advertising routing information to other OSPF routers. In turn, connectivity to HSRP virtual gateway at 10.10.4.1 was successfully tested from CORE-SW1, with a 100% packet success rate (5/5).

The switch at ASW-ADMIN1 was also tested for connectivity. There, an active management SVI for VLAN 10 with address 10.10.9.71/27 was found, and the two GigabitEthernet uplinks were functioning correctly. Nevertheless, a few connectivity issues were found when attempting to reach the RADIUS server and other hosts. In general, the core routing infrastructure was found to be properly connected, but individual hosts and connections needed further investigation.

7.2 Ping and Traceroute Tests

Several ping tests were performed to check the connectivity between the devices and specific hosts. Earlier tests of the HSRP virtual IP 10.10.4.1 from the PC have led to inconsistent results – namely, a 25% packet loss with very high response times ranging between around 681 ms to 2,141 ms. Later HSRP testing revealed that the virtual gateway IP address was still reachable – yet, the packet-loss phenomenon pointed to instability that was not consistent with the stable connectivity at the moment.

A stable test from CORE-SW1 to the HSRP virtual gateway 10.10.4.1 achieved 100% success (5/5 replies), with a minimum of 0 ms, an average of 121 ms and a maximum of 234 ms. RADIUS reachability remained intermittent during the available 802.1X tests. Traceroute testing was also completed: routing between authorised internal devices succeeded, while tracing from the restricted guest/untrusted side toward protected internal resources was unsuccessful as expected. These results support the intended routing and security restrictions while preserving the documented 802.1X/RADIUS limitation.

7.3 VLAN Testing

The VLAN configuration and operation were tested using VLAN, interface, MAC-address, ARP, and trunk information from the switches. VLAN 10 was configured as the MANAGEMENT VLAN, VLAN 20 was set as the ADMINISTRATION VLAN, and VLAN 90 was configured for the VOICE VLAN on the ASW-ADMIN1 switch. The switch’s connected access ports indicated that Fa0/1, Fa0/2, and Fa0/11 were assigned to VLAN 20, whereas Fa0/10 was assigned to VLAN 10. Additionally, the Fa0/11 had the voice VLAN 90 configured for the IP phone.

GigabitEthernet0/1 and GigabitEthernet0/2 on the ASW-ADMIN1 were up and configured as trunks and allowed VLANs 10, 20, and 90. The MAC-address table verified that the devices were learned in the appropriate VLANs. Specifically, the MAC addresses were dynamically learned in VLAN 10 via uplinks, whereas the phone’s MAC address was learned on the Fa0/11 VLAN 90. The ARP table on the DIST-SW1 also verified the active gateway and host entries for multiple VLANs, including VLANs 10, 20, 40, 50, 60, 70, 80, 90, 100, 110, 120, 130, and 140. Hence, VLAN segmentation and trunking were confirmed operational.

7.4 VPN Testing

The connectivity of the Virtual Private Network (VPN) was tested to ensure that secure communication was established between the main campus network and the remote network. The firewall configuration indicates that there is a site-to-site VPN relationship with the remote peer 172.16.0.6. The VPN service protected the traffic traveling between the 10.10.0.0/16 main-campus network and the 10.20.0.0/16 remote network. The test results show that the VPN security association was established, and the encrypted traffic was exchanged between the two networks. The correct operation of the VPN service is also evident from the number of IPsec packets. Specifically, there were outbound encrypted packets and inbound decrypted packets.

7.5 IPsec VPN Testing

The IPsec VPN was successfully tested on MAIN-FW1. The command show crypto isakmp sa revealed one active IKEv1 security association with remote peer 172.16.0.6 using a site-to-site (L2L) connection in QM_IDLE state. The show crypto ipsec sa output confirmed active encrypted traffic. One verified snapshot showed 1084 encrypted packets and 1143 decrypted packets with 0 send errors and 0 receive errors. A later snapshot also showed active encrypted/decrypted traffic with no errors; counter changes were treated as a reset or re-established security association rather than compared across sessions. The transform set used ESP-AES and ESP-SHA-HMAC. These results confirm that the IPsec tunnel was established and carried protected traffic between the main and satellite networks.

7.6 802.1X Testing

802.1X authentication was configured on ASW-ADMIN1 as part of the network security testing. First, AAA was configured using aaa new-model, and RADIUS server was defined as pointing to 10.10.9.102. The configured RADIUS authentication port was 1645, and the RADIUS shared key was <REDACTED>. Also, the switch was set to use aaa authentication dot1x default group radius which defines RADIUS as the authentication server for 802.1X.

Global 802.1X was enabled using dot1x system-auth-control. The administration PC access port, FastEthernet0/1, was set to access port in VLAN 20, and it was configured as an 802.1X authenticator using dot1x pae authenticator command. The command authentication port-control auto was deployed, placing the port into the automatic authentication mode. This means that the switch will need the connected endpoint to successfully authenticate before allowing normal network access.

The configuration was verified using show running-config command. The output confirmed presence of the AAA configuration, RADIUS server, global 802.1X enablement and the 802.1X settings on Fa0/1. However, the Packet Tracer switch did not support the show dot1x verification command, resulting in % Invalid input detected and % Unrecognized command outputs. Thus, it was not possible to confirm that there was an actual successful 802.1X authentication session using the 802.1X status command of the switch.

Testing also included connectivity to the RADIUS server. Initially, ASW-ADMIN1 received 0/5 replies when pinging 10.10.9.102, followed by a later test achieving 2/5 replies (40%). DIST-SW1 managed to achieve 4/5 replies (80%) to the same address. Thus, the RADIUS server was not consistently reachable from the administration switch. As a result, the configuration of 802.1X was completed, but it was not possible to confirm a successful end-to-end RADIUS authentication in the available testing.

7.7 Wi-Fi Testing

The Wi-Fi testing was performed as part of the University of Limpopo Smart Campus network verification. The wireless infrastructure consisted of the WLC-MAIN (10.10.9.70) and campus wireless and FlexConnect access-point infrastructure. The SDN monitoring script was able to detect and monitor the WLC, reporting it as [MONITORED]. The wireless design also incorporated separate VLANs for Campus Wi-Fi (VLAN 100) and Guest Wi-Fi (VLAN 110). The SDN policy specifically required the Guest Wi-Fi be separated from internal campus resources while still having access to the Internet with access to DNS and DHCP. This validated that wireless traffic was incorporated into the campus segmentation and security design.

The VoIP infrastructure was verified during testing. IP phones were registered with CME, with main-campus extensions 1001-1004 using 10.10.7.x addresses and satellite extensions 2001-2002 using 10.20.1.x addresses. An inter-site call from extension 1001 to 2001 was successfully established, and both endpoints showed CONNECTED with the G729 codec.

Data VLAN: VLAN 20 – ADMINISTRATION

Voice VLAN: VLAN 90 – VOICE

The switch configuration also confirmed that the IP phone on Fa0/11 used Voice VLAN 90 while the attached data device used VLAN 20. QoS configuration was verified on MAIN-EDGE-R1: WAN-QOS was applied outbound on Gi0/0, VOICE-RTP used strict priority with 20% bandwidth and DSCP EF, and ACL 120 matched the RTP UDP range. Packet Tracer did not increment the live VOICE-RTP counters during the active call, so live RTP classification was not demonstrated even though the QoS policy and attachment were verified.

7.8 VoIP/Video Testing

VoIP testing confirmed phone registration and a successful inter-site call between extensions 1001 and 2001. The network uses dedicated Voice VLAN 90 and a WAN QoS policy with strict priority, 20% bandwidth and DSCP EF for RTP traffic. The QoS policy was correctly configured and attached, but Packet Tracer did not show live RTP packets in the VOICE-RTP class counter. Video-stream quality was not benchmarked, so no unsupported jitter, throughput or video-quality result is claimed.

7.9 Security Testing

Security testing was focused primarily on network access control and 802.1X authentication procedures. The administration switch was configured to utilize AAA and a RADIUS server for authentication purposes. The latter’s IP address was set to 10.10.9.102, while 802.1X system authentication was enabled globally.

The administration PC’s switch port, Fa0/1, was configured to dot1x pae authenticator and authentication port-control auto. This setup established the switch as the authenticator and ensured that the endpoint would be authenticated before any access is granted. The port was also configured to retain spanning-tree portfast and spanning-tree bpduguard enable, providing an additional layer of protection that would be appropriate for an end-device access port.

The security testing was only partially conclusive, as the RADIUS server showed intermittent reachability from ASW-ADMIN1, and the Packet Tracer environment did not provide the show dot1x command that would be required to directly verify the authentication state. Consequently, the security configuration was implemented, but the successful authentication of a user/device through RADIUS was not conclusively demonstrated.

7.10 Performance Testing

Performance testing used ping response time and packet-loss measurements. Some first runs showed transient delay or packet loss while Packet Tracer completed ARP, convergence and protocol processing, so stable retests were also recorded. From the authorised Admin PC to WLC-MAIN (10.10.9.70), the stable retest achieved 0% packet loss with an average latency of 22 ms. Across the site-to-site VPN to 10.20.1.141, the stable retest achieved 0% loss with an average of 41 ms, and a later post-PAT retest averaged 23 ms with 0% loss. A reliable Mbps throughput benchmark was not obtained, so no unsupported throughput figure is reported.

7.11 Results and Screenshots

The testing has yielded some configuration and verification results that can be presented as evidence in the project. The OSPF neighbour output shows that CORE-SW1 has two fully established OSPF neighbours, 3.3.3.3 and 4.4.4.4. The OSPF routing table in turn shows that multiple networks are being learned

through both 10.10.255.10 and 10.10.255.14, which shows that multiple routing paths are available.

The HSRP testing has shown that VLAN 40 was using the virtual IP 10.10.4.1. CORE-SW1 successfully reached the virtual gateway with 5/5 packets. The HSRP output in turn shows the active router / standby-router information at different points during the testing which shows that the redundancy/failover configuration previously implemented and tested is present.

For VLAN / switching verification, ASW-ADMIN1 shows the VLAN 10, VLAN 20 and VLAN 90 as active with the appropriate set of ports assigned. Both uplinks are trunked, and the MAC-address table dynamically learns devices. The CDP output furthermore shows that Fa0/11 is connected to an IP phone.

Finally, the 802.1X running configuration provides evidence of the new security configuration that has been implemented. It verifies that aaa new-model, RADIUS configuration, aaa authentication dot1x default group radius, dot1x system-auth-control and the authenticator configuration is active on Fa0/1. However, the testing also verifies the caveats / limitations described in this project: that the show dot1x command is unsupported in the Packet Tracer switch, and the RADIUS connectivity is intermittent. Hence, the screenshots of the configuration outputs should be included along with the ping results to show which features are configured, in contrast to features that have been tested end-to-end.

7.12. NAT Testing

NAT/PAT was tested on MAIN-FW1. Before configuration, show run nat and show xlate showed no active NAT rule or translations. A CAMPUS-NET object for 10.10.0.0/16 was then configured with dynamic interface PAT using nat (inside,outside) dynamic interface. Verification with show nat showed the Auto NAT rule and translation hits increased from 0 to 7 and later to 13. The site-to-site VPN was retested after PAT was added and stable inter-site connectivity remained available, confirming that the simulated PAT configuration was processing ordinary outside traffic without breaking the working VPN.

7.13 SDN Controller Testing

The SDN controller was tested by running the Python automation program and ensuring that the defined network devices are successfully monitored. All the devices listed below were displayed as [MONITORED], including the DHCP/DNS server, WLC-MAIN, ASW-ADMIN1, DIST-SW1, DIST-SW2, CORE-SW1, and CORE-SW2. In addition, the SDN controller displayed the defined security policies and VLANs monitored by the controller. Finally, the SDN API test revealed that the Requests module was available. On the other hand, the HTTP API test displayed HTTP Status Code 200, which confirms that the controller’s HTTP connection was established.

8.Cost and Budget

8.1 Required hardware

The recommended smart campus network needs networking equipment and security equipment in order to run the main campus and the regional campus. The following are some of the hardware that will be required:

- Layer 3 Core switches – to form the network core.

- Layer 3 Distribution switches – used for routing and inter-VLAN communications.

- Access switches – for connecting user devices, IP phones, and Internet of Things (IoT).

- Routers – for site-to-site communications and for providing wide area network (WAN).

- Firewall – controls traffic between the internal network and external network.

- Wireless access points for Wi-Fi connection to the users, staffs, and visitors.

- Server for various functions including authentication/accounting/authorization and other university services.

- IP phones for Voice over Internet Protocol (VoIP).

- User computers and other network attached devices.

- Internet of Things (IoT) devices such as cameras, environmental sensors and smart class rooms.

- Appropriate Ethernet cables, fiber optics and network accessories.

It is designed using a hierarchical network topology that consists of core, distribution and access layer. This makes the network management easy and future expansion easier.

8.2 Network Simulation & Configuration Software

Network simulation and configuration software has been employed by the project to plan, develop and verify the network. The network device simulation and testing has been done using Cisco Packet Tracer.

The development has made use of technologies like:

- VLANs to segment the network.

- Inter VLAN routing.

- Dynamic routing using OSPF.

- Gateway redundancy using HSRP.

- IP address assignment using DHCP.

- Authentication using AAA/RADIUS.

- Port Based Network Access Control using 802.1X.

- Layer 2 Loop Avoidance using Spanning Tree Protocol.

- Trunking using IEEE 802.1Q.

8.3 Networking Equipment

The network design consists of several access switches that connect to the redundant distribution/core network infrastructure. For instance, the Administration access switch, ASW-ADMIN1, connects to the distribution switches via a redundant link.

The VLANs were used during network configuration in order to segregate traffic. Some VLAN examples are provided below:

- VLAN 10 - Management VLAN

- VLAN 20 - Administration VLAN

- VLAN 40 - Lecture-related network VLAN

- VLAN 90 - Voice VLAN

Residences VLANs, Library VLAN, Research VLAN, Servers VLAN, IoT VLANs, etc.

Trunk links were configured to transfer necessary VLANs between switches.

8.4 Estimated costs

However, the cost of implementing the proposed network would vary based on the number of buildings, users, access points, servers, and IoT devices to be used. Thus, a comprehensive business quote would be necessary prior to any physical implementation.

In this case, the design of the proposed solution is relatively inexpensive due to the fact that the implementation relies mainly on simulation rather than physical components.

In such a case, the cost of the physical implementation of the proposed solution would include the following components:

- Core and Distribution Switches

- Access Switches

- Routers and Firewalls

- Wireless Access Points

- Servers

- Fiber & Ethernet Cables

- IP Phones

- IoT Devices

- Installation of Equipment

- Software & Licensing

8.5 Cost-Saving measures

Some ways to cut down on the cost of implementation are as follows:

- Network simulation – Simulation can be done by using Cisco Packet Tracer before buying any hardware.

- Incremental design – The network can be incrementally expanded when more buildings and users come into picture.

- VLAN segmentation – VLAN provides efficient use of existing infrastructure while segregating the various types of traffic.

- Redundancy where necessary – Redundant devices are only installed for critical network components and not for every single piece of equipment.

- Centralized services – Authentication, DHCP, and DNS are centralized services and no need to install redundant servers in each building.

- Existing infrastructure reuse – Existing cabling and network equipment can be reused wherever possible.

- Simulation before deployment – Simulation ensures that there are no configuration problems beforehand.

9. Network Management and Maintenance

9.1 Network monitoring

Monitoring of the network is necessary to guarantee that the university network is always available, secure, and reliable.

In order to monitor and troubleshoot connectivity and performance in the network, some monitoring and troubleshooting commands were used. Some of the commands include the following:

- show ip interface brief

- show interfaces trunk

- show ip route

- show ip route ospf

- show ip ospf neighbor

- show ip protocols

- show arp

- show mac address-table

- show cdp neighbors

- show running-config

Connectivity of the network was also checked using ping command.

Verification of OSPF indicated that the core switch was able to establish FULL OSPF neighbor relationship with other devices. In addition, the routing table showed several OSPF routes via different paths.

Verification of HSRP indicated that virtual gateway 10.10.4.1 was set up and active and standby relationship was established for VLAN 40.

9.2 Backup and Recovery

The configurations of the networking devices must be backed up regularly. Backups must contain:

- Configurations of core switches.

- Configurations of distribution switches.

- Configurations of access switches.

- Configurations of routers.

- Configurations of firewalls.

- Configurations of DHCP and DNS servers.

- Configurations of AAA/RADIUS.

In case of failure of the network device, configuration of that device can be recovered on the new device. HSRP and redundant network routes assist in maintaining connectivity despite certain failures. Backup is stored securely and it is stored in more than one place.

9.3 Software updates

Network devices must be periodically reviewed for available firmware and software updates.

It is especially crucial to do security updates, as old software might be vulnerable to attacks.

This process needs to be done with care since it may have an impact on network availability. Prior to the updates:

- Make sure to back up your existing configuration.

- Determine compatibility and requirements.

- Arrange maintenance at an appropriate time.

- Conduct the updates.

- Perform connectivity tests after the updates.

- Document the changes.

9.4 Trouble Shooting

A systematic troubleshooting approach is recommended when there are problems in the network.There were some problems that came out during the project regarding connectivity. In some cases, ping was used but there were some packet losses and higher response times.

The troubleshooting steps include:

- Identifying the device or the affected service.

- Checking if the interfaces are physically up.

- Checking VLANs.

- Checking trunks.

- Checking IP addresses.

- Checking DHCP address assignment.

- Checking routing using the routing table.

- Checking neighbour relations in OSPF.

- Checking HSRP when redundancy exists.

- Using ping for testing connectivity.

- Checking ARP and MAC tables.

- Checking the RADIUS and authentication services.

- Fixing the configuration and re-testing.

9.5 Maintenance Plan

| Maintenance Task | Frequency | Reason for Performing This Task |
| --- | --- | --- |
| Status checking of network devices | Daily | To detect any failure/unavailable interfaces |
| Monitoring network performance | Daily | To detect any congestion/abnormal traffic |
| Security logging check | Daily | To detect any security incident |
| Configurations backup | Weekly | To facilitate recovery from equipment failure |
| Checking DHCP | Weekly | To ensure all devices get proper addresses |
| Checking VLAN and trunk | Weekly | To ensure segmentation is correct |
| Checking OSPF & HSRP | Weekly | To confirm routing and redundancy |
| User authentication check | Weekly | To find any authentication problems |
| Security/software update | Monthly/as required | To reduce any vulnerability |
| Backup recovery test | Monthly | To test whether backups are capable of being restored |
| Network design review | Quarterly | To find future capacity requirement |

10 Challenges Faced

10.1 Budget Challenges

10.2 Simulation Challenges

10.2.1DHCP Server

The problem that was observed during the testing was the incorrect DHCP address assignment. The DHCP service was not always assigning the correct IP addresses to different kinds of end devices such as wired PCs, IoT devices, and laptop computers using the wireless network.In this way, the network addressing problem emerges since the device receives the wrong IP address that belongs to the wrong address range or subnet. Besides that, receiving the wrong IP address means that the wrong default gateway is also assigned, which makes it impossible for the device to communicate with the right VLAN, gateway, server.Thus, it is not possible to consider the DHCP service completely validated in the course of simulation, and additional troubleshooting needs to be done regarding the DHCP pools, network addresses, default gateways, VLANs, DHCP relay configuration if any, trunk VLANs, and connection of the DHCP service to the network segments.

10.3 Hardware Challenges

One of the most significant hardware-related challenges during the design and implementation of the Smart Campus network was the complexity of configuring and establishing connectivity between different network devices. Switches that had to be appropriately configured included core switches, distribution switches, switches, routers, a firewall, wireless LAN controller and servers with the correct IP addressing, routing and connections. In addition, troubleshooting the design was complicated by the limitations of the Cisco Packet tracer software in which the design was tested. For instance, Cisco Packet Tracer lacks full support for enterprise SDN/open flow controller, hence the use of Python to monitor the traffic via HTTP API and not the enterprise controller. The issues included ensuring there were no connectivity problems between segments and that interfaces, routes, NAT, and VPN peers were appropriately configured. The final tests that were successfully performed included the monitoring of the enterprise network devices, traffic NAT PAT, and the establishment of a site-to-site IPsec VPN with encrypted and decrypted traffic.

10.4 Security Challenges

10.4.1 802.1X Testing Limitation

The proper functioning of 802.1X could not be fully tested and confirmed because the Packet Tracer simulation for switch did not allow all the necessary commands to carry out the test procedures fully. Although the configuration can be set up in Packet Tracer, it cannot provide the same type of confirmation that can be provided by using real Cisco switches and clients.

11. Conclusion and Recommendations

11.1 Summary of the proposed network

The proposed network was designed and successfully simulated to provide a secure, reliable, scalable and manageable campus network for the University of Limpopo. The designed network consists of multi-site, multilayered network infrastructure including Core Layer (CORE-SW1, CORE-SW2), Distribution Layer (DIST-SW1, DIST-SW2), Access Layer and Security Layer with MAIN-FW1 firewall. The main campus in Polokwane consists of Administration, Lecture Halls, Library and Research Labs connected to the other campus via WAN/Gateway routers through a secured IPsec VPN. The proposed network utilizes VLANs, Inter-VLAN routing, OSPF routing protocol, HSRP gateway redundancy, Network Address Translation (NAT), Access Control Lists (ACLs), 802.1X with RADIUS (10.10.9.102), DHCP/DNS (lms.unilim.local, aaa.unilim.local, sdn.unilim.local, mail.unilim.co.za) and Wireless LAN with Wireless LAN Controller (WLC) and Access Points (APs) for CAMPUS WIFI, GUEST WIFI, STAFF WIFI and IOT-WIFI. The designed network was implemented and tested using Cisco Packet Tracer.

11.2 Benefits of the design

The benefits of the proposed design include but are not limited to: First, a large coverage and capacity for more than 500 users with high performance and reliability for Wi-Fi, VoIP, video, and IoT traffic. Second, network security is achieved through Firewall, VLANs, ACLs (GUEST-ZT access list for VLAN 110, CCTV-ZT for VLAN 120 and IOT-ZT for VLAN 130), Wireless Protected Access version 2 (WPA2-PSK) and 802.1X authentication. Third, network redundancy and availability are achieved through utilizing two core and distribution switches, OSPF FULL neighbors and HSRP virtual IP address (10.10.4.1) for VLAN 40. Fourth, network manageability and operability are achieved through employing services such as DNS, DHCP, AAA, OSPF dynamic routing that does not require static routing configurations and IP addressing following the hierarchical model. In addition, the network was first tested using simulation to reduce costs before deployment.

11.3 Future improvements

Future improvements include deploying an enterprise SDN/OpenFlow controller instead of the Python HTTP API monitoring used in Packet Tracer, and validating 802.1X authentication on physical Cisco switches because the simulator did not support the required end-to-end status verification. QoS is already configured for Voice VLAN 90 using strict priority and DSCP EF; future testing should verify live RTP classification and voice-quality behaviour using physical equipment or a more capable simulator. Other improvements include expanding coverage to additional buildings and residences, adding more IoT devices and monitoring tools, and conducting regular backup and recovery tests in accordance with the maintenance plan.

12. References

Books

Odom, W. CCNA 200-301 Official Cert Guide.

Kurose, J. F., and Ross, K. W. Computer Networking: A Top-Down Approach.

Websites

Cisco Systems. Cisco networking documentation and configuration guides.

University of Limpopo. Official institutional website.

Documentation

Cisco ASA Firewall Configuration Guide

Cisco Wireless LAN Controller (WLC) Configuration Guide

Cisco Packet Tracer Documentation

Others

Packet Tracer simulation

Packet tracer show commands: show ip ospf neighbor , show ip route ospf , show standby , show access-lists , show interfaces trunk

13. Appendices

13.1 Network topology diagrams: Updated University Network Architecture Flow Chart (excluding Student Centre – includes only Admin, Lecture Hall, Library, Research Lab) and Network Design Packet Tracer logical topology diagram (showcasing CORE-SW1 (1.1.1.1), CORE-SW2 (2.2.2.2), DIST-SW1 (3.3.3.3), DIST-SW2 (4.4.4.4)

13.2 Configuration commands: OSPF configuration, HSRP configuration for VLAN 40 (virtual IP address 10.10.4.1), VLAN trunk configuration (native VLAN 1, allowed VLANs 10,20,90), 802.1X configuration (aaa new-model , aaa authentication dot1x default group radius , radius server 10.10.9.102 secret <REDACTED>), Access Control Lists (ACLs) GUEST-ZT, CCTV-ZT, IOT-ZT

13.3 IP addressing tables: Gateway IP addresses, Server IP addresses (DNS 10.10.9.100, AAA 10.10.9.102, Authorized host 10.10.9.105)

13.4 VLAN tables:

| VLAN | Name | Purpose |
| --- | --- | --- |
| 10 | MANAGEMENT | Network-device management |
| 20 | ADMINISTRATION | Administration users |
| 30 | ACADEMIC-STAFF | Academic/staff users |
| 40 | STUDENTS/LECTURE | Student and lecture-related network |
| 50 | LIBRARY | Library users and services |
| 60 | RESIDENCES | Student residences |
| 70 | RESEARCH | Research users and laboratories |
| 80 | SERVERS | University server network |
| 90 | VOICE | VoIP |
| 100 | CAMPUS-WIFI | Internal wireless users |
| 110 | GUEST-WIFI | Guest wireless users |
| 120 | CCTV | Security cameras |
| 130 | IOT | IoT devices |
| 140 | ENV-SENSORS | Environmental sensors |

13.5 Testing screenshots: OSPF neighbour FULL status, HSRP, VLAN/trunk verification, NAT/PAT translation counters, IPsec VPN counters, SDN controller Python [MONITORED] output, 802.1X/RADIUS configuration, VoIP call state, QoS policy attachment, Syslog output, and Zero-Trust ACL counters.

13.6 Additional technical information: Maintenance tasks table (daily, weekly, monthly), Hardware Use table, Cost and Budget, encountered challenges (DHCP server, Hardware configuration, 802.1X testing)